# Written Explanation

Robotic Arm Joint Health Monitoring System — Bonus Challenge

## 1. Why These Safety Thresholds for the Immediate-Stop Check

The immediate safety thresholds were chosen to reflect the physical limits of industrial robotic arm components operating under continuous assembly-line conditions:

• Torque → 85 Nm: Standard 6-axis robotic arm actuators in the 10–20 kg payload class are typically rated to 80–90 Nm peak torque. At 85 Nm, the actuator is operating at its structural limit; any further load risks gearbox stripping or shaft deformation. This value is set below the theoretical maximum to provide a safety margin before catastrophic failure.

• Temperature → 80°C: Industrial servo motor windings are commonly rated to an insulation class  of  'F'  (155°C  maximum  winding  temperature),  but  the  external  joint  housing  —  the observable measurement point — should not exceed 80°C during normal duty cycles. Above this, lubricants in the gearbox begin to thin rapidly, bearing races expand beyond tolerance, and the  heat-dissipation  pathway  to  the  motor  core  becomes  ineffective.  80°C  is  therefore  a conservative housing-surface limit that protects the internal components.

• Vibration  →  9.5  mm/s:  ISO  10816-3  (machinery  vibration  standards)  classifies  vibration above 7.1 mm/s as "unsatisfactory" for smaller industrial machines and above 11.2 mm/s as "dangerous." The 9.5 mm/s threshold sits in the "alarm" zone — severe enough to indicate imminent bearing damage or fastener fatigue, but below the point where the arm must already have suffered permanent damage.

Critically, the immediate-stop check is implemented entirely in safety_check.py with no import of or call to the ML model. It will operate correctly even if wear_detector.py is broken, untrained, or completely absent.

## 2. How the ML Prediction and Maintenance Log Combine into a Final Decision

The system treats the ML wear prediction and the maintenance log as two independent evidence channels. Neither alone is sufficient to trigger maintenance scheduling, because each can produce false positives in isolation:

• The ML model might predict wear on an arm that was recently serviced and is performing normally — it sees the operating hours but not the service history.

• A maintenance note might use words like "monitoring closely" after a minor lubrication, which scores some concern points without representing a real problem.

The combination logic in decision_engine.py uses a simple AND gate for scheduling and an OR gate for monitoring:

| ML Signal | Maintenance Signal | Decision |
| --- | --- | --- |
| Wear | Concern | “ SCHEDULE_MAINTENANCE ” |
| Wear | No concern | “ MONITOR ” |
| Normal | Concern | “ MONITOR ” |
| Normal | No concern | “ SAFE ” |

This  means  the  system  requires  corroboration  from  both  channels  before  committing  to  a maintenance schedule, which reduces false maintenance dispatches. A single uncertain signal only escalates to monitoring — a lighter action that increases sensor poll frequency without stopping production.

## 3. How the Spoofed-Sensor Defense Works

Attack model: An attacker (or a faulty sensor) feeds an artificially low torque reading to one joint while the arm  is actually under high  mechanical stress. The  intent  is to  make the joint appear healthy so the safety check passes and no maintenance is scheduled — masking a real problem.

Defense —  Method  1  (Cross-sensor  correlation):  Under  real  high-stress  conditions,  torque, temperature, and vibration all rise together due to basic physics: high force → increased friction → heat, and bearing irregularity →  vibration.  If temperature ≥  65°C AND vibration ≥  6.0 mm/s but torque ≤ 40 Nm on the same joint, the combination is physically inconsistent. The system flags this joint as suspicious and escalates to EMERGENCY_STOP.

Defense — Method 2 (Intra-arm statistical outlier): For a single arm's three joints operating under the same load profile, torque values should be broadly similar (accounting for joint position in the kinematic chain). If one joint's torque has a z-score below _2.5 relative to the other joints while overall arm stress is elevated (mean temperature ≥ 60°C or mean vibration ≥ 5 mm/s), that joint's reading is an anomaly — and an anomalously low reading under stress is the spoofing signature.

What this attack would and would not catch:

•YES Would catch: A single-joint torque spoof where the attacker only manipulates the torque channel and leaves temperature/vibration unchanged.

•YES Would catch: A sudden step-change drop in torque for one joint while others remain elevated.

•NO Would miss: A sophisticated multi-channel attack that simultaneously spoofs torque, temperature, and vibration to consistent low values. In this case, all three sensors agree, and the cross-sensor check cannot distinguish a spoof from a genuinely idle joint.

•NO Would  miss: A very slow, gradual torque drift attack that stays within the z-score threshold across many readings.